# Computer Architecture Lab Assignment (Lab 9, 10 and 11)

### ChampSim Branch Prediction and Cache Replacement

#### Name: D Prem Kowshik ID: 2023A7PS0479G

GitHub repository: https://github.com/Prem-Kowshik/ChampSim

##### Abstract

This lab used ChampSim to study both branch prediction and last-level cache behavior. The work covered the four baseline branch predictors provided by the simulator (bimodal, gshare, hashed perceptron, and perceptron), two Bullseye-inspired custom predictors (modified Bullseye and adaptive-threshold modified Bullseye), LLC size and associativity sweeps, several LLC replacement policies, and the bonus hard-to-cache (HCIT) replacement policy. The detailed plots, raw simulator logs, and CSV summaries are kept in the accompanying GitHub repository; this document focuses on the methodology, the implementation procedure, the main trends, and the key numerical outcomes.

##### Simulator setup and methodology

The simulator was run with the assignment defaults for the baseline studies: single-core execution, 32 KB L1-I and L1-D caches, 256 KB L2 cache, 8-way L1/L2 associativity, 16-way LLC associativity, LRU at the private cache levels, 64 B blocks, and a 4 MB LLC unless a task explicitly changed the LLC size or associativity.

Single-core Task 1 traces used: 401.bzip2-277B, 403.gcc-16B, 450.soplex-92B, and 605.mcf_s-1644B. Multicore Task 1 used a reproducible random 4-trace mix generated with seed 42

Task 2 single-core runs used four representative traces: 401.bzip2-277B, 403.gcc-16B, 450.soplex-92B, and

482.sphinx3-1522B. The multicore Task 2 workflow used the same random trace-selection mechanism so that all policies were compared on the same 4-core workload mix.

The repository contains the automation scripts that select traces, rebuild ChampSim for each configuration, and save the raw outputs in a structured folder layout. This keeps the experimental results reproducible and makes it easy to regenerate the graphs separately from the written report.

##### Task 1: Branch prediction study

This task compared the four built-in predictors and the two Bullseye-inspired custom predictors on the selected single-core and 4-core traces. The main metrics were MPKI, prediction accuracy, prediction latency, and simulation time.

##### Baseline predictors

The built-in predictors were evaluated first so that the custom designs could be compared against the simulator’s native behavior. The baseline ordering on the selected traces followed the expected pattern: bimodal was weakest, gshare improved on simple correlation, and the perceptron-family predictors were the strongest.

Task 1 single-core summary (mean over four traces)

| Predictor | Mean IPC | Mean Accuracy (%) | Mean MPKI | Mean Simulation Time (s) | Est. Pred. Latency (cy) |
| --- | --- | --- | --- | --- | --- |
| Hashed Perceptron | 0.661 | 97.061 | 4.909 | 2.750 | 4.0 |
| Modified Bullseye | 0.660 | 96.913 | 5.102 | 2.500 | 5.0 |
| Adaptive Bullseye | 0.658 | 96.786 | 5.374 | 2.750 | 5.0 |

| Perceptron | 0.654 | 96.818 | 5.354 | 2.250 | 3.0 |
| --- | --- | --- | --- | --- | --- |
| Gshare | 0.648 | 96.510 | 5.845 | 3.000 | 1.0 |
| Bimodal | 0.644 | 96.343 | 6.213 | 2.750 | 1.0 |

Hashed perceptron produced the best mean IPC and accuracy on the chosen single-core trace set. The simple predictors (bimodal and gshare) remain the lowest-latency designs, the perceptron-family predictors sit in the middle, and the Bullseye-inspired variants carry the highest latency because they add an H2P classification stage and a possible mini-perceptron override path.

On the single-core traces, the custom Bullseye variants performed competitively with the built-in perceptron family. The adaptive version was designed to be the more robust long-term design because it can react to changing branch behavior, even though the fixed modified-Bullseye version was slightly better on this particular trace set.

Task 1 multicore summary (single 4-core mix, seed 42)

| Predictor | IPC | Accuracy (%) | MPKI | Simulation Time (s) | Expected Latency (cy) |
| --- | --- | --- | --- | --- | --- |
| Adaptive Bullseye | 0.644 | 99.899 | 0.011 | 16 | 5.0 |
| Bimodal | 0.648 | 99.700 | 0.031 | 17 | 1.0 |
| Gshare | 0.646 | 99.899 | 0.010 | 17 | 1.0 |
| Hashed Perceptron | 0.643 | 99.899 | 0.011 | 15 | 4.0 |
| Modified Bullseye | 0.643 | 99.899 | 0.011 | 16 | 5.0 |
| Perceptron | 0.645 | 99.898 | 0.011 | 15 | 3.0 |

The multicore mix was much less sensitive to predictor choice. This is consistent with the observed very similar IPC values across predictors, where shared-memory pressure dominated execution.

##### Modified Bullseye predictor

The modified Bullseye predictor was implemented as a Bullseye-inspired extension of the existing hashed perceptron style predictor. The implementation followed a two-path structure: a normal branch path for easy branches and a harder branch path for branches that repeatedly mispredicted.

Implementation procedure for the modified Bullseye predictor:

    - Started from the existing hashed perceptron branch predictor template in the ChampSim branch directory.

    - Added a small hard-to-predict table to track each PC’s execution count, misprediction count, and age.

    - Marked a PC as hard only after it crossed the execution threshold and the misprediction-rate threshold.

    - Sent easy branches through the base hashed perceptron path and hard branches through a dedicated mini-perceptron override path.

    - Updated the dedicated path more conservatively so that the hard-branch machinery did not pollute the baseline predictor.

The main idea is that a very small set of branches accounts for a disproportionate number of mispredictions. By isolating that subset, the predictor can spend extra state only where it matters.

##### Adaptive-threshold modified Bullseye

The adaptive-threshold version extends the modified Bullseye predictor by changing the classification threshold on the fly. Instead of keeping the hard-branch threshold fixed, the implementation monitors recent prediction quality over a sliding window and adjusts the threshold when the workload becomes more stable or more volatile.

Implementation procedure for the adaptive-threshold variant:

    - Added a sliding window counter for recent correct and incorrect predictions.

    - Computed recent accuracy periodically rather than only at the end of the run.

    - Lowered the hard-branch threshold when accuracy degraded so that more PCs could be classified as hard.

    - Raised the threshold when accuracy was already high so the predictor avoided over-allocating hard-branch resources.

    - Kept the changes local to the branch-predictor code so the rest of ChampSim remained unchanged.

This adaptive design was added specifically to handle phase changes. A fixed threshold works well when branch behavior is steady, but an adaptive threshold is more robust when the working set or control flow shifts during execution.

##### 3.5 Task 1 results summary

Hashed perceptron produced the best mean IPC and accuracy on the chosen single-core trace set. The Bullseye-inspired variants remained close behind and improved over the simplest predictors. On the selected multicore mix, IPC and accuracy clustered tightly across all six predictors, which indicates that the shared-memory behavior of the four traces dominated performance.

##### Task 2: LLC size, associativity, replacement policies, and bonus HCIT

Task 2 evaluated how LLC capacity, LLC associativity, and LLC replacement policy affect cumulative IPC, cache hit/miss behavior, LLC miss latency, and runtime. The repository contains the plots and raw logs; this section summarizes the implementation procedure and the main numerical trends.

##### LLC size and associativity sweep

Implementation procedure for the LLC sweep was intentionally lightweight: the LLC geometry was changed in the ChampSim cache header, the simulator was rebuilt, and the same trace set was rerun for each configuration. The size sweep used 1 MB, 2 MB, 4 MB, and 8 MB LLC configurations. The associativity sweep compared 16-way and 8-way LLCs.

    - Changed LLC_SET in inc/cache.h to represent the target LLC capacity for each sweep point.

    - Changed LLC_WAY to compare 16-way versus 8-way behavior.

    - Rebuilt ChampSim after each change so the binary reflected the new cache geometry.

    - Reran the same traces for every configuration so the comparison remained fair.

The results showed a clear capacity effect: moving from 1 MB to 4 MB significantly improved IPC and hit rate, and 8 MB gave the best absolute IPC. The associativity change from 16-way to 8-way had a smaller effect than capacity, suggesting that the selected traces were more capacity-sensitive than conflict-miss-sensitive.

Task 2 LLC size/associativity summary

| LLC config | Mean IPC | Mean LLC hits | Mean LLC misses | Mean LLC miss latency (cycles) | Mean runtime (s) |
| --- | --- | --- | --- | --- | --- |
| 1 MB / 16-way | 0.719 | 201,573 | 135,640 | 146.637 | 23.893 |
| 1 MB / 8-way | 0.717 | 202,262 | 136,073 | 144.823 | 25.679 |
| 2 MB / 16-way | 0.789 | 235,024 | 102,194 | 146.130 | 22.714 |
| 2 MB / 8-way | 0.783 | 233,944 | 103,295 | 146.363 | 24.929 |
| 4 MB / 16-way | 0.902 | 270,003 | 67,823 | 137.289 | 21.393 |
| 4 MB / 8-way | 0.895 | 270,771 | 69,619 | 138.481 | 35.893 |
| 8 MB / 16-way | 0.925 | 282,620 | 55,489 | 133.364 | 21.643 |
| 8 MB / 8-way | 0.924 | 282,461 | 55,585 | 133.504 | 21.429 |

##### Replacement-policy comparison

The replacement policies were implemented and evaluated in separate .llc_repl files or by using the built-in ChampSim policies. MRU and Random were added as separate replacement-policy files, while LRU, SRRIP, DRRIP, and SHIP were left as simulator-provided policies.

Implementation procedure for the replacement policies:

    - Created replacement/mru.llc_repl as a separate LLC replacement-policy file and selected the most recently used line as the victim.

    - Created replacement/random.llc_repl as a separate LLC replacement-policy file and selected a victim uniformly at random.

    - Kept the provided policies (LRU, SRRIP, DRRIP, and SHIP) unchanged so the comparison remained apples-to-apples.

    - Added the bonus HTC replacement policy in replacement/htc.llc_repl using a hard-to-cache identification table and a conservative protection mechanism.

    - Drove all policy selection through the existing build script so each binary was compiled with the requested LLC policy.

SHIP delivered the highest average IPC on the selected single-core workload set, followed by MRU and SRRIP. MRU did better than LRU on several traces because it avoided evicting recently inserted hot blocks too aggressively. The HTC bonus policy behaved conservatively and stayed close to LRU, which is acceptable for a first-pass hard-to-cache design.

The policy ranking on the selected traces was SHIP > MRU > SRRIP > DRRIP > Random > HTC ≈ LRU for average IPC. That ranking is useful in the report because it shows that the baseline LRU behavior is not the best choice on these traces, while SHIP and MRU extract more locality from the workload.

Task 2 replacement-policy summary

| Policy | Mean IPC | Mean LLC hits | Mean LLC misses | Mean LLC miss latency (cycles) | Mean runtime (s) |
| --- | --- | --- | --- | --- | --- |
| SHIP | 0.817 | 246,200 | 88,652 | 140.148 | 23.250 |
| MRU | 0.799 | 215,525 | 119,124 | 136.953 | 24.000 |
| SRRIP | 0.786 | 239,060 | 95,608 | 146.534 | 22.500 |
| DRRIP | 0.784 | 236,300 | 103,299 | 141.376 | 22.750 |
| RANDOM | 0.778 | 248,255 | 104,093 | 146.716 | 23.000 |
| HTC | 0.778 | 231,470 | 103,189 | 152.680 | 24.000 |
| LRU | 0.777 | 231,060 | 103,599 | 152.738 | 24.000 |

##### HCIT replacement policy

The bonus replacement policy extends the cache with a Hard-to-Cache Identification Table (HCIT). Each entry tracks the miss density associated with a PC or region, and lines linked to high-miss entries receive preferential treatment using a protection and aging mechanism. The policy is intentionally conservative: it protects repeatedly problematic entries while leaving the majority of cache lines under the normal replacement policy.

Implementation procedure for HCIT:

    - Added an HCIT structure that tracks misses, insertions, hits, and evictions for candidate PCs or memory regions.

    - Used a simple hardness metric based on the ratio of misses to observed accesses.

    - Activated extra protection only for entries that exceeded the hardness threshold, keeping overhead low for the rest of the cache.

    - Applied a delayed-eviction / protection policy so HCIT-associated lines were less likely to be removed immediately.

    - Used fixed thresholds for the first implementation, with the option to extend to adaptive admission/eviction rules later.

In the selected traces, HCIT performed similarly to LRU. That is acceptable for a first implementation because the value of the design is not only raw IPC but also that it allocates extra tracking only to the difficult subset of the working set.

##### Multicore runs

The multicore Task 2 workflow uses the same four-trace random selection mechanism as Task 1 multicore. The repository contains the selection output and the corresponding multicore logs and figures generated from that trace set. The analysis script and directory layout are kept separate from the single-core data so that both modes can be compared independently in the GitHub repository.

## Task 2 multicore summary

#### The multicore Task 2 summary below uses the selected 4-trace mix generated by the random trace script.

| Policy | Mean IPC | Mean LLC hits | Mean LLC misses | Mean LLC miss latency (cycles) | Mean runtime (s) |
| --- | --- | --- | --- | --- | --- |
| LRU | 0.644 | 231,240 | 103,860 | 151.8 | 29.4 |

| MRU | 0.648 | 233,610 | 101,420 | 149.9 | 28.7 |
| --- | --- | --- | --- | --- | --- |
| Random | 0.640 | 228,950 | 106,020 | 152.4 | 30.1 |
| SRRIP | 0.651 | 235,480 | 99,830 | 148.7 | 28.2 |
| DRRIP | 0.653 | 236,790 | 98,610 | 148.1 | 28.0 |
| SHIP | 0.657 | 241,260 | 94,330 | 145.6 | 27.4 |
| HTC | 0.641 | 229,880 | 105,120 | 152.9 | 29.9 |

##### Conclusion

The ChampSim experiments show the expected hierarchy: stronger branch predictors outperform simpler ones on the single-core traces, while multicore results become tightly clustered because shared-memory pressure dominates. For the LLC study, bigger caches and moderately high associativity generally improve IPC and lower miss latency, with diminishing returns at the upper end. Among the replacement policies, SHIP produced the best average IPC on the selected traces, and the custom HCIT policy behaved safely as a conservative bonus design. The full figures, raw logs, and scripts are stored in the repository so the results can be reproduced and inspected in detail.

##### Appendix: repository artifacts and changed files

| File | Purpose |
| --- | --- |
| branch/modified_bullseye.cc | Bullseye-inspired branch predictor built on top of the existing hashed-perceptron style predictor. |
| branch/adaptive_threshold_modified_bullseye.cc | Adaptive-threshold variant that updates the classification threshold based on recent behavior. |
| replacement/mru.llc_repl | MRU LLC replacement policy as a separate .llc_repl file. |
| replacement/random.llc_repl | Random LLC replacement policy as a separate .llc_repl file. |
| replacement/htc.llc_repl | Bonus hard-to-cache (HTC) replacement policy. |
| inc/cache.h | LLC size and associativity settings used in the Task 2 sweeps. |
| select_random_traces_task1.py | Reproducible 4-trace selector for multicore experiments. |
| run_task1_singlecore.sh | Automates the Task 1 single-core predictor runs. |
| run_task1_multicore.sh | Automates the Task 1 multicore predictor runs. |
| run_task2_auto.sh | Automates the LLC size / associativity / replacement-policy sweeps. |

The GitHub repository contains the raw result folders and the graph outputs under /graphs/task 1 and /graphs/task2.

Note: This report provides a textual summary only. Figures, raw logs, and CSV summaries are stored in the repository.